1. BLDC Driver Specifications

| Driver Model | # SBDCM-03 |
| --- | --- |
| Continuous Output Current | 3A |
| Peak Current | 5A |
| Rated Input Voltage | AC 100 ~ 220 V 15% |
| Controllable Signal | 1. Run/Stop 3. Direction 2. Speed |
| Control Method | 3 Phase PWM |
| Protection | Over Current(OC) |
| LED Display | 1. Power LED 2. OC LED |
| Option | 1. Speed Control 2. Hall Sensor Signal |
| Operating Temperature | 0C ~ 50C |

2. Connector Specifications

Input, Hall, Motor, UI

| Name | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Input | NC | AC | AC |  |  |  |
| Hall Motor | W |  |  | Hw | Hv | Hu |
|  | V | U |  |  | Vcc | G |

U/I & Connection (5267-6)

| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- |
| GND | CW | CCW | EXT_VR | SPEED OUT | ALARM | +12V |

3. Functions

| 전원 표시 | - 전원을 연결하면 PW LED가 점등된다. |
| --- | --- |
| Over Current 표시 | - 출력단의 단선 및 장시간 동안의 과부하 등으로 과도한 전류가 흐르는 경우OC LED가 점등되고, 3번이 High(12V)로 되며, 출력을 차단한다. |
| 속도 조정 | - GND 단자와 CW/CWW 단자가 Short 된 상태에서 정면 속도 볼륨 조정기로 속도 조정이 가능합니다. |
| Run / Stop | - GND 단자와 CW/CCW 단자가 Open 이면 Stop , Short 시키면 Run - GND 단자와 CW/CCW 단자가 Short된 상태에서 정면 Run/Stop 스위치로 Run/Stop 이 가능합니다. ※ STOP은 메인 전원이 차단된 상태가 아닙니다. |
| 회전 방향 선택 | - GND 단자가 CW 단자와 Short면 CW방향으로 회전, CCW 단자와 Short면 CCW 방향으로 회전. ※ 고속에서 정역을 바꾸면 운전조건에 따라 위험한 경우가 있으므로 Maker와 협의 할 것 |

4. Designation of Driver

Front                                   Rear

5. Dimensions(mm)