1. BLDC Driver 사양

| Driver 형명 | # SBDSMS-03 |
| --- | --- |
| Continuous Output Current | 4A |
| Peak Current | 10A |
| Rated Input Voltage | 단상 100 ~ 220 V 15% |
| Controllable Signal | 1. Run/Stop 4. Reset 2. Speed 5. Direction 3. Brake |
| Control Method | 3 Phase PWM |
| Protection | 과부하 / 오결선 / 모터이상 |
| LED Display | 1. Power LED 2. ALARM LED |
| Operating Temperature | 0C ~ 50C |

2. Connector 사양

Input, Hall, Motor

| 명칭 | 사양 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Input | 5557D-04 | AC | NC | AC | FG |  |  |
| Hall Motor | 5557D-12 | W |  |  | Hw | Hv | Hu |
|  |  | V | U |  |  | Vcc | G |

U/I 및 결선  (5267-11)

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GND | Dir. Out | ALARM | Speed Out | ALARM Reset | F/R | Brk | R/S | GND | Speed | +5V |

10k

3. 기능 설명

| 전원 표시 | - 전원을 연결하면 PW LED가 점등된다. |
| --- | --- |
| 모터이상 | - 모터가 연결되지 않은 경우 또는 신호이상인 경우 ALARM LED가 점멸한다. |
| Over Current 표시 | - 출력단의 단선 및 장시간 동안의 과부하 등으로 과도한 전류가 흐르는 경우ALARM LED가 점등되고, 3번이 LOW(GND)로 되며, 출력을 차단한다. |
| 속도 조정 | - 10번 단자에 외부에서 전압을 인가하는 경우 최대 5V이하로 해야 함 - U/I Connector의 9,10, 11에 가변저항이 부착되어있는 경우 시계방향이 속도가 증가하는 방향임 |
| ALARM Out | -과부하 오결선 모터이상 경고 신호출력 |
| Speed Out | -모터의 속도에 따른 펄스출력 |
| Run / Stop | - R/S 단자를GND와 Open 하면 Stop, Short 시키면 Run |
| 회전 방향 선택 | - 모터 RUN 상태에서 F/R 단자를GND와 Short/Open하면 회전방향이 바뀐다. ※ 고속에서 정역을 바꾸면 운전조건에 따라 위험한 경우가 있으므로 Maker와 협의 할 것 |
| Dir. Out | - CW / CCW 방향에 대해 Open Collector 신호가 출력됨. |

4. 외부 제어기와의 인터페이스

Direction Out

ALARM Out

Speed Out

10V

Micom

포토커플러

Vcc

Micom

4.7K

F/R,

R/S

BRAKE

F

속도 지령

U/I Connector의 10번 Speed단자에 아날로그 전압을 인가한다.

5. 외각 도면

6. 드라이버 결선도